- WY6005M_Configuration_Command_List

| Version | Date | Update Content |
| --- | --- | --- |
| V1.1 | 2025-09-19 | Added one-line/one-point output command : AT+SPAD_OUTPUT_LINE_DATA |
| V1.0 | 2025-09-10 | Compiled command list |

- Command Description

  - Command Classification

Category 1: Only supports basic query functions;

AT+QUERY_DEVICE_INFO?\n

AT+QUERY_DEVICE_ID?\n

AT+QUERY_LIDAR_RESOLUTION?\n

AT+QUERY_LENS_INFO?\n

Category 2: Supports both query and configure functions.

| query | configure |
| --- | --- |
| AT+DEVICE_INDEX?\n | AT+DEVICE_INDEX=1\n (Optional parameters 1, 2, 3, 4) |
| AT+STREAM_CONTROL?\n | AT+STREAM_CONTROL=1\n (Optional parameters 0,1) |
| AT+STREAM_DATA_TYPE?\n | AT+STREAM_DATA_TYPE=1\n(Optional parameters 0,1,2,3,4) |
| AT+SPAD_FRAME_MODE?\n | AT+SPAD_FRAME_MODE=1\n (Optional parameters 0,1) |
| / | AT+SPAD_TRIG_ONE_FRAME=1\n (Only effective in one-frame mode, and this parameter only supports being set to the value 1) |
| AT+SPAD_OUTPUT_LINE_DATA?\n | AT+SPAD_OUTPUT_LINE_DATA=1,1,64\n (There are three parameters in total: Parameter 1 has a value range of 0-8; Parameters 2 and 3 have a value range of 1-64; and the value of Parameter 2 must be less than or equal to that of Parameter 3) |

command list

| command name | RW | parameter | Description |
| --- | --- | --- | --- |
| Basic Commands |  |  |  |
| QUERY_DEVICE_INFO | R | 1 | The Return: content includes software version information and hardware version information in string format. Send command: AT+QUERY_DEVICE_INFO? Return: example: QUERY_DEVICE_INFO="WY6005_MCU, BUILD: Feb 8 2025 10:44:42, res=[64,8]" |
| QUERY_DEVICE_ID | R | 1 | Parameter type: BUF_HEX (hexadecimal buffer), length: 12 Bytes. Example as follows: Send command: AT+QUERY_DEVICE_ID? Return: example:QUERY_DEVICE_ID=1A1E31353133310459414731 |
| DEVICE_INDEX | RW | 1 | Used to query/set the device index number, parameter type: uint8 (8-bit unsigned integer): Device index value description: 1=Left device, 2=Front device, 3=Right device, 4=Rear device; Query command: AT+DEVICE_INDEX? Set command: AT+DEVICE_INDEX=1 |
| QUERY_DEVICE_SN | R | 1 | Parameter type: BUF_HEX (hexadecimal buffer),length: 8 Bytes. Example as follows: Send command: AT+QUERY_DEVICE_SN? Return: example: QUERY_DEVICE_SN=12345678ABCDEF90 |
| QUERY_LIDAR_RESOLUTION | R | 2 | The Return: value contains 2 integer-type parameters, with detailed explanations as follows: •Parameter 1: Radar horizontal point count •Parameter 2: Radar vertical point count |
| QUERY_LENS_INFO | R | 3 | The Return: value is the lens optical information, containing 3 float-type values, with detailed explanations as follows: •Parameter 1: X-axis optical parameter •Parameter 2: Y-axis optical parameter •Parameter 3: Z-axis optical parameter |
| STREAM_CONTROL | RW | 1 | Parameter •?:query •0:OFF •1:ON |
| STREAM_DATA_TYPE | RW | 1 | Output data format: •?:Query current format •0:Peak data •1:Distance data •2:Depth data •3:Point data(Point cloud) •4:Debug data (Peak + histogram) |
| SPAD_TRIG_ONE_FRAME | W | 1 | In one-frame mode, the device will output the next frame of data after this command is sent. |
| SPAD_FRAME_MODE | RW | 1 | Used to query/set the device output mode, with parameter value descriptions as follows: Parameter : •1: one-frame output mode •0: continuous output mode |
| AT+SPAD_OUTPUT_LINE_DATA | RW | 3 | Used to query/set the output of only the specified range of data in the specified row, with parameter descriptions as follows: •Parameter 1: Number of the row to be output (value range: 0-8); where 0 means outputting all 8 rows of data •Parameter 2: Start point of the specified data range in the row to be output •Parameter 3: End point of the specified data range in the row to be output Constraint for Parameters 2 and 3: 1 ≤ Parameter 2 ≤ Parameter 3 ≤ 64 |

- Command Format Description

Command prefix: "AT+"

Query device parameter: AT+CMD?

Query parameter format: AT+CMD=?

Send/Set parameter: AT+CMD=arg1,arg2,arg3 // parameter separated by ","

| Parameter Type | Sub type | Example |
| --- | --- | --- |
| CAT_VAR_INT_DEC | INT8,INT16,INT32 | 11 |
| CAT_VAR_UINT_DEC | UINT8,UINT16,UINT32 | 22, |
| CAT_VAR_NUM_HEX | HEX8, HEX16, HEX32 | 0x2233, |
| CAT_VAR_BUF_HEX | HEXBUF | 1234abcd, |
| CAT_VAR_BUF_STRING | STRING | ”1234abcd” |
| Double/float | Not Supported | Alternative solution: Pass floating-point values as strings, and the receiver converts them to Double/float type. Example: AT+dev_offset_a="-3737.2750351004"\n |

Device response rules:

Append “OK” to the end of Return:ed data on success, otherwise Return: “ERROR”

Commands are case-insensitive (compatible with upper and lower cases).

CMD command example:

Send:（ format: Param 1 (int8_t), Param 2 (HEX16), Param 3 (Hex String), Param 4 (String)）

AT+CMD=11,0x2233,1234abcd,”1234abcd”\n

Return:

OK

Query data example:

Send:

AT+CMD?\n

Return:

CMD=11,0x2233,1234abcd,”1234abcd” OK

Query data format example:

Send:

AT+CMD=?\n

Return:

CMD=<speed:INT32[RW]>,<address:HEX16[RW]>,<buffer:HEXBUF[RW],<string:STRING[RW]> OK

Command error examples:

Send:

AT+ERRORCMD=?\n

Return:

ERROR

- Operation Description

  - Serial Port Configuration Description

Basic serial port parameter configuration:

Baud rate: 921600

Data bits: 8

Stop bits: 1

Parity bit: NONE

Sending settings: Check the "Automatic escape character parsing" option

Command sending rule: A newline character (\n) must be added at the end of the AT command

Example: AT+STREAM_CONTROL=1\n

  - SAVE CONFIG

Parameters configured via setting commands only take effect until the next device reboot and will be lost after reboot.

Send “AT+SAVE_CONFIG\n” saves all previously set parameters to flash, which the device loads to work after reboot to ensure the parameters persist across restarts.

  - stream control operation

stream control switch status control: 1=On, 0=Off.

Switch 0 (Off): No data output;

Switch 1 (On):

Continuous output mode: Output data continuously;

One-frame output mode: Send the latest frame of data after receiving the trigger command (AT+SPAD_TRIG_ONE_FRAME=1\n), then stop output.

Operation rule: Turn off the flow control switch before querying/setting parameters, and turn it on again after completion to resume normal operation.

  - Turn off radar frame data output, Send :AT+STREAM_CONTROL=0\n		Return: OK

  - Turn on radar frame data output, Send :AT+STREAM_CONTROL=1\n		Return: OK

  - Query Basic Information

  - Turn off lidar frame data output,Send: AT+STREAM_CONTROL=0\n	Return: OK

  - Query parameter:

- Query device info

- Send:AT+QUERY_DEVICE_INFO?

- Return::QUERY_DEVICE_INFO="WY6005_MCU, BUILD: Feb  8 2025 10:44:42, res=[64,8]"

- Query device id

- Send:AT+QUERY_DEVICE_ID?\n

- Return::QUERY_DEVICE_ID=1A1E31353133310459414731

- Query device SN

- Send:AT+QUERY_DEVICE_SN?

- Return::QUERY_DEVICE_SN=12345678ABCDEF90

- Query lidar resolution

- Send:AT+QUERY_LIDAR_RESOLUTION?\n

- Return::QUERY_LIDAR_RESOLUTION=64,8	(The first parameter is the horizontal point count, and the second parameter is the vertical point count.)

- Query lens optical information

- Send:AT+QUERY_LENS_INFO?\n

- Return::QUERY_LENS_INFO="2.2","0.27","0.62"

  - Turn on radar frame data output,Send: AT+STREAM_CONTROL=1\n	Return: OK

  - Query/Set Output Mode

The output mode is divided into continuous output mode and single-frame output mode. In continuous output mode, data is output continuously; in single-frame output mode, only the latest frame of data is sent after a specific AT command is sent.

Parameter 0 indicates the output mode is continuous output mode, and Parameter 1 indicates the output mode is single-frame output mode.

    - Query Output Mode

  - Turn off lidar frame data output,Send: AT+STREAM_CONTROL=0\n	Return: OK

  - Query Output Mode:	Send: 	AT+SPAD_FRAME_MODE?\n

- Return:: 	SPAD_FRAME_MODE=0

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return: OK

    - Set Output Mode

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return: OK

  - Set Output Mode,Send: 		AT+SPAD_FRAME_MODE=1\n	Return: OK

  - Save configuration,Send:	AT+SAVE_CONFIG\n			Return: OK

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return: OK

  - Outputting One Frame in One-frame Mode

Send “AT+SPAD_TRIG_ONE_FRAME=1\n” via the serial port, and the device will transmit the latest frame of data. This command is only valid when the output mode is set to one-frame mode ( SPAD_FRAME_MODE=1).

  - Query/Set Data Output Format

Parameter 0 indicates the output data format is peakdata (raw data output);

Parameter 1 indicates the output data format is distancedata (distance data output);

Parameter 2 indicates the output data format is depthdata (depth data output);

Parameter 3 indicates the output data format is pointdata (point cloud data output);

Parameter 4 indicates the output data format is debugdata (debug data output).

    - Query Data Output Format

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return: OK

  - Query Data Output Format,	Send:	AT+STREAM_DATA_TYPE?\n

- Return::	STREAM_DATA_TYPE=0

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return: OK

    - Set Data Output Format

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return: OK

  - Set Data Output Format,Send:	AT+STREAM_DATA_TYPE=1\n		Return:	OK

  - Save configuration,Send:		AT+SAVE_CONFIG\n			Return: 	OK

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return:	OK

  - Query/Set Device Index

When a host device is connected to multiple WY6005 devices, the index number identifies the physical position of the current device. The initial definition is as follows: parameter 1/2/3/4 corresponds to the left/front/right/rear position respectively.

    - Query Device Index

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return: OK

  - Query Device Index,			Send:AT+DEVICE_INDEX?\n

- Return: DEVICE_INDEX=0

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return: OK

    - Set Device Index

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return:	OK

  - Set Device Index:				Send:AT+DEVICE_INDEX=1\n		Return:	OK

  - Save configuration,				Send:AT+SAVE_CONFIG\n		Return: 	OK

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return:	OK

  - Query/Set the In-Row Range Data for Specified Output

    - Query the In-Row Range Data for Specified Output

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return:	OK

  - Query the In-Row Range Data for Specified Output:

- Send: 	AT+SPAD_OUTPUT_LINE_DATA? \n

- Return: 	SPAD_OUTPUT_LINE_DATA=1,1,64

  - Turn on radar frame data output,Send:  AT+STREAM_CONTROL=1\n	Return:	OK

    - Set the In-Row Range Data for Specified Output

  - Turn off lidar frame data output,Send:  AT+STREAM_CONTROL=0\n	Return:	OK

  - Set the In-Row Range Data for Specified Output.	Example:

- All data of row 1,Send: AT+SPAD_OUTPUT_LINE_DATA=1,1,64\n		Return:	OK

- Points 5-15 of row 1,Send: AT+SPAD_OUTPUT_LINE_DATA=1,5,15\n	Return:	OK

- Point 32 of row 1,Send: AT+SPAD_OUTPUT_LINE_DATA=1,32,32\n 	Return:	OK

- All 8 rows of data,Send: AT+SPAD_OUTPUT_LINE_DATA=0,1,64\n 	Return:	OK

- Note: When Parameter 1 is 0, all 64*8 data will be output by default, and Parameters 2 and 3 are invalid at this time.

  - Save configuration,Send:		AT+SAVE_CONFIG\n			Return: 	OK

  - Turn on radar frame data output,Send: AT+STREAM_CONTROL=1\n	Return:	OK