WY6005M_Operation Instructions for One-line /One-point Output

- Notes

  - Serial Port Configuration

- Baud Rate: 921600

- Data Bit: 8

- Stop Bit: 1

- Parity Bit: NONE

  - Data Format

one-line/one-point output only supports the “pointdata” format, which is the factory default configuration of the device and ready for use without modification

- one-line/one-point output Config

  - Configuration Rules

Three parameters are required: Row ID, In-row Start Point ID, In-row End Point ID.

The area array is 64 columns and 8 rows.

For one-line output configuration:

1≤Row ID≤8

1≤In-row Start Point ID≤In-row End Point ID≤64

Example: To configure the output of points 1 to 64 in row 4, the command is AT+SPAD_OUTPUT_LINE_DATA=4,1,64\n

For one-point output configuration:

1≤Row ID≤8

1≤In-row Start Point ID=In-row End Point ID≤64

Example: To configure the output of point 32 in row 4, the command is AT+SPAD_OUTPUT_LINE_DATA=4,32,32\n

(Note: For one-point/one-line output, the Row ID cannot be set to 0; setting it to 0 means outputting all data of the 64-column and 8-row array,and Parameters 2 and 3 will be invalid.)

  - Configuration Operation Methods

Send AT commands via the serial port with the following steps:

    - Configure to One Frame Output Mode/Continuous Output Mode

- Configure to One Frame Output Mode

- Close stream output:

- Send command: AT+STREAM_CONTROL=0\n		Return: \nOK\n

- Configure to One Frame Output Mode:

- Send command: AT+SPAD_FRAME_MODE=1\n		Return: \nOK\n

- Open stream output:

- Send command: AT+STREAM_CONTROL=1\n 		Return:\nOK\n+Device info

- Configure to Continuous Output Mode

- Close stream output:

- Send command: AT+STREAM_CONTROL=0\n		Return: \nOK\n

- Configure to Continuous Output Mode:

- Send command: AT+SPAD_FRAME_MODE=0\n		Return: \nOK\n

- Open stream output:

- Send command: AT+STREAM_CONTROL=1\n 		Return:\nOK\n+Device info

- （Continuous output mode is factory default)）

    - Configure to One Point Output Mode/One Line Output Mode

- Configure to One Point Output Mode

- Close stream output:

- Send command: AT+STREAM_CONTROL=0\n		Return: \nOK\n

- Configure to One Frame Output Mode:

- For example, to output the data of point 32 in row 1:

- Send command AT+SPAD_OUTPUT_LINE_DATA=1,32,32\n	Return:\nOK\n

- Open stream output:

- Send command: AT+STREAM_CONTROL=1\n 		Return:\nOK\n+Device info

- Configure to One Line Output Mode

- Close stream output:

- Send command: AT+STREAM_CONTROL=0\n		Return: \nOK\n

- Configure to One Frame Output Mode:

- For example, to output the data of point 1-64 in row 1:

- Send command AT+SPAD_OUTPUT_LINE_DATA=1,1,64\n	Return:\nOK\n

- Open stream output:

- Send command: AT+STREAM_CONTROL=1\n 		Return:\nOK\n+Device info

- Note: When Parameter 1(ROW ID) is set to 0, all 64×8 data will be output by default, and Parameters 2 and 3 will be invalid at this time.

    - Save Config

- Close stream output:

- Send command: AT+STREAM_CONTROL=0\n		Return: \nOK\n

- Save Config:

- Send command: AT+SAVE_CONFIG\n				Return:\nOK\n

- Open stream output:

- Send command: AT+STREAM_CONTROL=1\n 		Return:\nOK\n+Device info

- (When this command is executed successfully, the currently modified configuration will be saved to Flash. The device will load the modified configuration after power-off and restart; if this command is not executed, the device will run with the original configuration after power-off and restart.)

- Data Format Description

  - Data Format Description for One Frame Output Mode

In Single Frame Output Mode, a specific AT command is required for data acquisition each time; the returned data includes the AT command response and the data itself.

| the AT command response | \nOK\n |
| --- | --- |
| data | x |
|  | y |
|  | z |
|  | grey |

    - Data Format Description for One Point Output Mode

Example: Point 32 in Row 4（AT+SPAD_OUTPUT_LINE_DATA=4,32,32\n）

Send command:AT+SPAD_TRIG_ONE_FRAME=1\n

Return：

    - Data Format Description for One Line Output Mode

Example: Point 1-64 in Row 4（AT+SPAD_OUTPUT_LINE_DATA=4,1,64\n）

Send command: AT+SPAD_TRIG_ONE_FRAME=1\n

Return：

  - Data Format Description for Continuous Frame Output Mode

In Continuous Output Mode, each frame of data consists of two parts: a frame header and the data.

frame header :

| Meaning | Data bytes | Example（Little-endian） |
| --- | --- | --- |
| Tag (Fixed as "wyld") | 4 | 77 79 6C 64 |
| Width (Output Data Width) | 2 | 40 00 |
| Height (Always 1 in one-line/one-point Mode) | 2 | 01 00 |
| Framesize (Frame Data Size) | 4 | 00 02 00 00 |
| Linesize (Line Data Size) | 2 | 00 02 |
| Datatype（Data Type） | 2 | 03 00 |
| Frameidx (Frame Index, Incrementing) | 4 | D8 64 00 00 |
| Device_index (Device Index Number) | 1 | 01 |
| Reserved Segment | 11 | 00 00 00 00 00 00 00 00 00 00 00 |

DATA(Datatype = pointdata)：

| Meaning | Data bytes |
| --- | --- |
| x | 2 |
| y | 2 |
| z | 2 |
| grey | 2 |

    - Data Format Description for One Point Output Mode

Example: Point 32 in Row 4（AT+SPAD_OUTPUT_LINE_DATA=4,32,32\n）

Returns the frame header and data,continuous output.

    - Data Format Description for One Line Output Mode

Example: Point 1-64 in Row 4（AT+SPAD_OUTPUT_LINE_DATA=4,1,64\n）

Returns the frame header and data,continuous output.